AQUA HEATER

| TAF offers Titanium Immersion Heaters for Hatcheries with the design of: Principle to maintain the salty water / seawater at required temperature in the tanks of maturation, larvae breeding & artimia. For longer services & durability under severe electrolytic condition. With the quality of 100% immersiable. Heaters are made from 99.9 % pure Titanium pipe confirming to ASTM B337 Gr. 1, so it ensures no self-corrosion. Heaters are designed to be functional with automatic temperature controller system. Transparent & corrosion resistance caps are fitted snugly on heater & assembled with indicator lamp, to protect the system. Standard electric cable with PVC / Metal socket attached. Electro-hydraulic leak tested, it provides shock proof of current & leak proof of liquid. Capacity of the heaters can be enlarged upon request. |  | DETERMINATION OF HEATER REQUIREMENT: Estimate total kilowatt-hour requirement. Roughly speaking, to increase 1 liter of water (freshwater and salt water) 1degree, 1 kcal of energy is needed, which represents 1.16 x 10 kw.hr heating requirement. For example, to raise 20 tons water 5 degree up, the heating requirement is 1.16 x 10 x 5 x 20,000 = 116 (kw.hr). - Determine the desired time to reach the desired water temperature. For example, for the above condition in 5 hours. Determine the type of heater and number of heaters For example, if 2 kW heaters are selected, to raise 20 tons water 5degre up in 5 hours, roughly 12 heaters are required (116 ÷2 ÷5 = 11.6). |
| --- | --- | --- |
| Specification & Price: #### RATING; #### VOLTAGE; ##### AVAILABLITY; PRICE. IN Rs.; US$. 1.0 kW; 220 / 1 ph; 2.5 Cm dia x 40 Cm Long; 1,800.00; 33.00. 2.0 kW; 220 / 1 ph; 2.5 Cm dia x 60 Cm Long; 2,400.00; 45.00. 440 / 3 ph; 2.5 Cm dia x 60 Cm Long; 2,400.00; 45.00. 3.0 kW; 220 / 1 ph; 2.5 Cm dia x 80 Cm Long; 2,850.00; 50.00. 440 / 3 ph; 2.5 Cm dia x 80 Cm Long; 2,850.00; 50.00. Note: ** Capacity can be enlarged upon request. ** Prices are based on ex-Works, Chennai in packed condition. ** Prices are valid for the minimum order quantity of 100 Nos. ** Prices can be changed without any further notice. |
| Temperature controller: Using titanium lined thermocouples to resist from saltiness. Temperature controllers are available in the following standards with digital / analog display.  | ###### Specification of Temp. Controller: Voltage; Phase; Hz; Amp; KW. 220/440; 1 & 3; 50; 30/50; 6/9/18/30. |

TAF 10